Tender text SINEAX AM2000

Transparent power quality and energy consumption monitoring

The SINEAX AM2000 is a compact instrument for measuring and monitoring in power distribution systems. It excels in display quality and intuitive operation. The devices provide a wide range of functionalities which may even be extended by optional components.

The modern, multi-lingual design allows operating and parameterizing the device via website or the high-resolution color TFT display (800x480px). The integrated webserver provides remote acquisition, visualization and analysis of measured data. No additional software is required.

Condition monitoring

- Provision of all voltages, currents, powers, power factors every 10/12 cycles

- Extended reactive power analysis

- Variance of system variables with time reference

Energy flow analysis

- Power  mean-values with trend-analysis for preventing load-peaks

- Load profile storage with fluctuation range (min/max) per interval

- Active /reactive energy class 0.5S  (HT/LT, demand/delivery), configurable resolution

- Further mean-values and meters with selectable base quantity

Power quality analysis

- Harmonic analysis acc. IEC 61000-4-7

- Assessment of voltage and current imbalance

- Phasor and phase sequence display for connection check

- Optional disturbance recorder for PQ events (voltage dip, swell, interruption) acc. IEC 61000-4-30 Ed.3 with RMS1/2 and waveform recordings

Hardware options

Up to 4 modular extensions: Analog outputs ±20mA (2/4), Relay outputs (2), Digital inputs (4) active / passive, GPS time synchronization, Fault-current monitoring (residual or earth current), temperature monitoring (Pt100, PTC), IEC61850, Profinet I/O

Communication

- Optional: Ethernet (Modbus/TCP, http/https, NTP, SFTP, SYSLOG), IEC61850, Profinet I/O, Modbus/RTU

- Scheduled data export to SFTP server (option Ethernet required)

Security (option Ethernet required)

- Flexible Role Based Access Control

- Encrypted communication via HTTPS

- Audit Log records all security related procedures, support for SYSLOG for transfer to central network monitoring server

- Client whitelist: IP address list for restricting authorized computers

- Digital firmware signature: Prevents that manipulated firmware gets into the device via update

Further features

- 3 current and 3 voltage channels with 0.2% accuracy, direct measurement or via transformers

- Measurement category 600V, CAT III (voltage), 300V,CAT III (current)

- Data logger with16 GB data memory (option)

- Sampling rate: 18kHz

- Monitoring and alarming via limit values and monitoring functions

- For panel mounting 144x144mm

- Identical operation as devices of the PQ/CU/DM series

Manufacturer

Camille Bauer Metrawatt AG

Aargauerstrasse 7

CH-5610 Wohlen

https://www.camillebauer.com/