Tender text SINEAX DM5S

Programmable multi-transducer for heavy-current quantities

For the concurrent measurement of multiple quantities in power systems

Variants	Universal version for all systems or

all kind of balanced load systems or

single-phase and balanced-load 4-wire systems

Systems	Single-phase, split phase, 3-/4-wire systems balanced / unbalanced

Meas. inputs	400V Ph-N (max. 480V), 693V Ph-Ph (max. 832V)

1…5A (max. 7,5A)

45…65Hz

Meas. interval	Programmable: 4,8,16,32…1024 cycles, synchronized to power frequency

Meas. category	CAT III

Base quantities	TRMS: Current neutral current (calculated), voltage, frequency, active / reactive power, zero displacement voltage, load factors, bimetal current with slave pointer, current and voltage average values, harmonics up to 31st and THD V/I via Modbus

Energy meters	32 meters, up to 16 tariffs, active energy Cl.0.5S, reactive energy Cl.2

Programmable meter resolution

Uncertainty	V,I: 0,15%; P,Q,S: 0,2%; f: 0,01Hz (for meas. interval 8 cycles)

Outputs	0,1,2,3,4 analog outputs ±20mA, galvanically isolated

Communication	USB configuration interface, optional Modbus/RTU interface

Power supply	Universal 24…230V DC, 100…230V AC

Housing	for top hat-rail mounting 110x70x70mm (HxWxD) 
IP30 (housing), IP20 (terminals), UL94 V-0

Type	SINEAX DM5S + feature coding

Manufacturer	Camille Bauer AG

Aargauerstrasse 7

CH-5610 Wohlen

https://www.camillebauer.com