Tender SIRAX BM1250

Multifunctional measuring device for network parameters

For the measurement, monitoring and analysis of power networks

Netform			Single phase, 2-wire-Net

Three phase, 3-/4-wire-Net unbalanced load

Measurement input			Nominal voltage 57,5…346,4VLN, 100…600VLL

Nominal currant 1A / 5A

45…65Hz

Protection class			II

Measurement category	CATIII 300V

Basic values			Current (I1,I2,I3,IN,ISystem); Voltage (U1,U2,U3,USystem,U12,U23,U31;)

Frequency; Power (P1,P2,P3,ΣP,Q1,Q2,Q3,ΣQ,S1,S2,S3,ΣS);

Power factor (PF1,PF2,PF3,ΣPF); Phaseangle (Cosϕ1,Cosϕ2,Cosϕ3,ΣCosϕ)

Counter	Active energy Cl.0,2s, reactive energy Cl.2, apparent energy Cl.1, meters

Network analysis			THD_U, THD_I, Harmonics up to 31st harmonic

Uncerainty 			U,I: 0,2%; P,S: 0,2%; Q: 1,0%; THD: 3,0%

Inputs/Outputs			Pulse output

Interface			Modbus/RTU (RS485)

Measured value display	LCD-Display

Parameterization			via pushbutton or Modbus/RTU

Power supply			100…550V AC/DC

Housing	Housing for panel mounting 96x96 (front), installation depth 52mm or 70mm, IP54 (front), IP20 (housing/terminals), UL94 V-0

Type			SIRAX BM1250

Manufacturer			Camille Bauer Metrawatt AG

Aargauerstrasse 7

CH-5610 Wohlen

http://www.camillebauer.com