Tender SIRAX BT5300

Programmable transducer for frequency

Netform			Single phase

Measurement input			Nominal voltage 57…500V

Nominal frequency 45…55Hz, 48…52Hz, 55…65Hz, 45…65Hz

Protection class			II

Measurement category	CATIII ≤300V

Measurement uncerainty 	0,2%

Outputs			Linearisation linear / kinked

0…20mA / 4…20mA or 0…10V

Interface			Modbus/RTU (RS485)

Measured value display	LCD-Display

Parameterization			via pushbutton or Modbus/RTU

Power supply			60…300V AC/DC

Housing	Housing for DIN Rail mounting 43,75x65,5x106,5mm,

IP50 (front), IP20 (housing/terminals), UL94 V-0

Type			SIRAX BT5300

Manufacturer			Camille Bauer Metrawatt AG

Aargauerstrasse 7

CH-5610 Wohlen

http://www.camillebauer.com