Tender SIRAX BT5400

Programmable transducer for power

Netform			Single phase

3-phase 3-wire balanced or unbalanced load

3-phase 4-wire balanced or unbalanced load

Measurement input			Nominal voltage 57…500V

Voltage converter primary value 57…400V

Nominal current 1…5A

Current converter primary value 1…9999A

Nominal frequency 45…55Hz, 48…52Hz, 55…65Hz, 45…65Hz

Protection class			II

Measurement category	CATIII ≤300V

Measurement uncerainty 	0,2%

Power factor			0,5%

Outputs			Linearisation linear / kinked

0…20mA / 4…20mA or 0…10V

Interface			Modbus/RTU (RS485)

Measured value display	LCD-Display

Parameterization			via pushbutton or Modbus/RTU

Power supply			60…300V AC/DC

Housing	Housing for DIN Rail mounting 43,75x65,5x106,5mm,

IP50 (front), IP20 (housing/terminals), UL94 V-0

Type			SIRAX BT5400

Manufacturer			Camille Bauer Metrawatt AG

Aargauerstrasse 7

CH-5610 Wohlen

http://www.camillebauer.com